General Requirements -  Laser Safety Class 3b and Class 4 lasers

- Laser registration submitted to Laser safety officer

- Warning Signs outside laser controlled area

- Warning Light outside laser controlled area

- General Laser Safety Training for all users /spectators  (contact LSO for information 313-577-1200)

- Laser Specific Training for all users and documented within lab SOP (given by PI or designated laser user)

- Doors closed and access restricted during open beam laser use.

- Standard Operating Procedure (SOP) present and followed

- Alignment Procedures present and followed

- Key to laser is controlled

- Enclose as much of the beam path as possible.

- Open beam ?– put up barriers or curtains to contain the laser light

- Place beam stop at ends of the useful beam path.

- Sit or stand well above or below the laser beam path.

- Firmly mount the laser to ensure beam does not stray from intended path.

- Use proper eye protection for open beam lasers.

- Remove all unnecessary reflective surfaces from the beam path.